Week 1 — Subnetting & IP Addressing

Tool: Cisco Packet Tracer

## Scenario

A small office. One router, one switch, four departments: HR, Sales, Engineering, Management. They must be isolated at layer 3. You have 192.168.10.0/24.

## Subnet plan

Divide the /24 into 4 equal subnets before opening Packet Tracer.

| Dept | Network | Mask | Gateway | Hosts |
| --- | --- | --- | --- | --- |
| HR | 192.168.10.0/26 | 255.255.255.192 | 192.168.10.1 | .2 – .62 |
| Sales | 192.168.10.64/26 | 255.255.255.192 | 192.168.10.65 | .66 – .126 |
| Engineering | 192.168.10.128/26 | 255.255.255.192 | 192.168.10.129 | .130 – .190 |
| Management | 192.168.10.192/26 | 255.255.255.192 | 192.168.10.193 | .194 – .254 |

## Build

- 1 router, 1 switch, 4 PCs — one PC per department

- Assign each PC the correct IP, mask, and gateway from the table above

- Configure the router so each subnet has a working gateway

- Verify: every PC can ping its own gateway

## Break

Change the Engineering PC's subnet mask to 255.255.255.0. Ping the gateway and observe what happens.

## Fix

Use only ipconfig, ping, and tracert to find what is wrong. Fix it. All 4 pings must work again.

## Explain — write this before you finish

- What does a subnet mask do to an IP address?

- Why does /26 give 62 usable hosts, not 64?

- What broke when you changed the mask to /24 — and why?

- What is the broadcast address of 192.168.10.128/26?

- Why can a PC in subnet 1 not ping subnet 3 without a router?

## Done when

- All 4 PCs ping their gateway — screenshot

- Engineering broken ping — screenshot

- ipconfig showing the fault — screenshot

- All 4 pings restored — screenshot

- 5 written answers — your own words